Project: Creating an Optimized Windows Master Image

# 1. Project objectives

The goal was to implement a customized, lightweight, secure Windows system image that was already configured with all the software needed for the company's work environment. This image needed to allow for rapid deployment to new machines without reinstalling and reconfiguring basic applications, drivers, and settings each time.

# 2. Theoretical first plan

The project was initially to follow the following steps:

1. Install a clean Windows (without bloatware).

2. Install all necessary software manually or automatically.

3. Update the system and install all drivers.

4. Use Sysprep to generalize the system image.

5. Capture the image with a tool like DISM or Macrium Reflect.

6. Restore this image to other machines that are the same or close.

# 3. Steps completed

During the project, several adjustments were necessary. Here is the actual process:

1. Clean Windows 11 installation without bloatware.

2. Change of strategy to Atlas OS, a lighter version of Windows for more performance.

3. Creating a PowerShell script using Winget to automate software installation:

- Browsers: Chrome, Firefox, Edge

- Office automation: Office 365

- Communication: Microsoft Teams

- Development: Visual Studio Code, Docker Desktop, PuTTY, Git

- PDF: Adobe Acrobat Reader

- Networks: Wireshark, FileZilla

4. Fixed the issue with the PowerShell execution policy (Set-ExecutionPolicy).

5. Formatting a USB drive to FAT32 via PowerShell and DiskPart for compatibility with Macrium.

6. Creating a Macrium Reflect bootable key.

7. Capture de l’image système (semi-universelle car sans sysprep).

8. Restore test successfully on a blank machine.

# 4. Problems met and solutions

• PowerShell script would not launch because of runtime policies ➝ solution:

(Set-ExecutionPolicy RemoteSigned).

• The JSON file used for Winget was non-compliant ➝ Solution: Deprecated the JSON format and used a PowerShell script directly.

• Sysprep does not work with Atlas OS (changes too deep) ➝ Solution: Switch to a semi-universal image via Macrium Reflect.

• Macrium refused some USB formats ➝ manual reformatting in FAT32 or NTFS.

• Difficulty in automating the launch of the script on the first connection ➝ bypassed by manually including the script if necessary.

# 5. Tools and utilities used

• Windows 11 Pro: Basic system for initial testing.

• Atlas OS: A lightweight version of Windows, ideal for reducing latencies and optimizing performance.

• Winget: Modern package manager built into Windows 10/11 to automate app installation.

• PowerShell: For installation automation, USB drive management, and system configuration.

• Macrium Reflect Free: Reliable and easy-to-use system image capture and restoration tool.

• DiskPart: A command-line tool for partitioning disks.

• External hard drive: System image storage (.mrimg).

# 6. Readjustments and decisions taken

• Sysprep is deprecated due to its incompatibility with Atlas OS (missing or corrupted components).

• Adoption de Macrium Reflect pour la capture d’une image semi-universelle (fonctionne même sans sysprep).

• Switching from a Winget JSON script to a PowerShell script to work around JSON validation errors.

• Added a post-restore verification step with Macrium if the machine does not boot.

# 7. Sources and Resources Used

Software:

• https://www.macrium.com/reflectfree

• https://atlasos.net/

• https://learn.microsoft.com/en-us/windows/package-manager/winget/

• https://learn.microsoft.com/en-us/powershell/

YouTube Videos:

- https://www.youtube.com/watch?v=NI9TldX9L4I

Forums and Communities:

- https://www.reddit.com/r/AtlasOS

- https://forums.macrium.com/

- https://superuser.com/